# esp32_a_test.py

# Flow: Spin wheel → Press START → RFID reads color → LED on → Shuffle → ESP-NOW to B

from machine import Pin, SPI

import time, random, network, espnow

# -- ESP-NOW setup -------------------------------------------------------------

sta = network.WLAN(network.STA_IF)

sta.active(True)

sta.disconnect()

en = espnow.ESPNow()

en.active(True)

B_MAC = b'\xA4\xF0\x0F\x75\xA4\xDC'   # ESP32 B MAC address

en.add_peer(B_MAC)

def send_msg(msg):

en.send(B_MAC, msg)

print("ESP-NOW TX: " + msg)

def wait_result(timeout_ms=8000):

deadline = time.ticks_ms() + timeout_ms

while time.ticks_diff(deadline, time.ticks_ms()) > 0:

result = en.recv(0)

if result[0] is not None:

msg = result[1].decode().strip()

print("ESP-NOW RX: " + msg)

return msg

time.sleep_ms(50)

return "TIMEOUT"

# -- Indicator LEDs (show chosen colour) ---------------------------------------

led_yellow = Pin(25, Pin.OUT)

led_green  = Pin(26, Pin.OUT)

led_blue   = Pin(27, Pin.OUT)

# -- Slot LEDs (one per slot) --------------------------------------------------

slots  = [Pin(13, Pin.OUT), Pin(14, Pin.OUT), Pin(15, Pin.OUT)]

button = Pin(5,  Pin.IN,  Pin.PULL_UP)

# -- RFID ----------------------------------------------------------------------

spi = SPI(2, baudrate=1000000, polarity=0, phase=0,

sck=Pin(18), mosi=Pin(23), miso=Pin(19))

rst = Pin(4,  Pin.OUT)

cs  = Pin(21, Pin.OUT)

cs.value(1)

rst.value(0); time.sleep_ms(50); rst.value(1); time.sleep_ms(50)

def _write(reg, val):

cs.value(0)

spi.write(bytes([(reg << 1) & 0x7E]))

spi.write(bytes([val & 0xFF]))

cs.value(1)

def _read(reg):

cs.value(0)

spi.write(bytes([((reg << 1) & 0x7E) | 0x80]))

val = spi.read(1)

cs.value(1)

return val[0]

def _set(reg, mask): _write(reg, _read(reg) |  mask)

def _clr(reg, mask): _write(reg, _read(reg) & ~mask)

def _to_card(cmd, send):

irq_en   = 0x77 if cmd == 0x0C else 0x12

wait_irq = 0x30 if cmd == 0x0C else 0x10

_write(0x02, irq_en | 0x80)

_clr(0x04, 0x80); _set(0x0A, 0x80); _write(0x01, 0x00)

for b in send: _write(0x09, b)

_write(0x01, cmd)

if cmd == 0x0C: _set(0x0D, 0x80)

i = 2000

while True:

n = _read(0x04); i -= 1

if not (i and not (n & 0x01) and not (n & wait_irq)): break

_clr(0x0D, 0x80)

recv = []; bits = 0; stat = 1

if i and (_read(0x06) & 0x1B) == 0x00:

stat = 0

if n & irq_en & 0x01: stat = 1

elif cmd == 0x0C:

n = _read(0x0A); lb = _read(0x0C) & 0x07

bits = (n - 1) * 8 + lb if lb else n * 8

n = max(1, min(n, 16))

recv = [_read(0x09) for _ in range(n)]

return stat, recv, bits

def request():

_write(0x0D, 0x07)

s, _, b = _to_card(0x0C, [0x26])

return (0 if s == 0 and b == 0x10 else 1)

def anticoll():

_write(0x0D, 0x00)

s, uid, _ = _to_card(0x0C, [0x93, 0x20])

if s == 0 and len(uid) == 5:

chk = 0

for i in range(4): chk ^= uid[i]

if chk != uid[4]: s = 1

return s, uid

def rc522_init():

_write(0x01, 0x0F); time.sleep_ms(50)

_write(0x2A, 0x8D); _write(0x2B, 0x3E)

_write(0x2D, 30);   _write(0x2C, 0)

_write(0x15, 0x40); _write(0x11, 0x3D)

_set(0x14, 0x03)

TAGS = {

(0x3C, 0x0B, 0xD5, 0x06, 0xE4): "YELLOW",

(0x7A, 0x9A, 0x5F, 0x10, 0xAF): "GREEN",

(0xAA, 0x8A, 0x74, 0x10, 0x44): "BLUE",

}

# -- Helpers -------------------------------------------------------------------

def show_indicator(colour):

led_yellow.value(1 if colour == "YELLOW" else 0)

led_green.value( 1 if colour == "GREEN"  else 0)

led_blue.value(  1 if colour == "BLUE"   else 0)

def all_slots_off():

for s in slots: s.off()

def read_colour_now():

"""Try to read colour for up to 3 seconds after button press."""

print("Reading wheel color...")

deadline = time.ticks_ms() + 3000

while time.ticks_diff(deadline, time.ticks_ms()) > 0:

if request() == 0:

s, uid = anticoll()

if s == 0:

colour = TAGS.get(tuple(uid), None)

if colour:

return colour

time.sleep_ms(100)

return None  # nothing detected in 3 seconds

def shuffle(colour):

print("Shuffling for " + colour)

winning = random.randint(0, 2)

idx = random.randint(0, 2)

# Intro: chosen colour in ALL 3 slots — blink 3 times then hold

for _ in range(3):

for s in slots: s.on()

time.sleep_ms(400)

all_slots_off()

time.sleep_ms(150)

for s in slots: s.on()

time.sleep_ms(700)

all_slots_off()

time.sleep_ms(250)

# Phase 1: FAST - hard to follow!

for _ in range(22):

all_slots_off(); slots[idx % 3].on()

time.sleep_ms(45); idx += 1

# Phase 2: SLOW - looks like it might stop (suspense)

for _ in range(8):

all_slots_off(); slots[idx % 3].on()

time.sleep_ms(random.randint(220, 320)); idx += 1

# Phase 3: FAST again - keeps you guessing!

for _ in range(22):

all_slots_off(); slots[idx % 3].on()

time.sleep_ms(random.randint(40, 55)); idx += 1

# Phase 4: MEDIUM until winning slot then instant BLACKOUT

while idx % 3 != winning:

all_slots_off(); slots[idx % 3].on()

time.sleep_ms(random.randint(95, 125)); idx += 1

# Flash winning slot briefly then instant BLACKOUT

slots[winning].on()

time.sleep_ms(50)

all_slots_off()

print("BLACKOUT! Winning slot: " + str(winning + 1))

return winning

# -- Startup -------------------------------------------------------------------

rc522_init()

show_indicator(None)

all_slots_off()

print("Spin the wheel then press START!")

# -- Main loop -----------------------------------------------------------------

while True:

# Wait for button press

if button.value() == 0:

time.sleep_ms(30)

if button.value() == 0:

while button.value() == 0: time.sleep_ms(10)

print("START pressed - reading color...")

send_msg("START")          # tell B to beep for start

colour = read_colour_now()

if colour:

print("Color: " + colour)

send_msg("COLOR:" + colour)   # sync NeoPixel on B immediately

# Blink indicator LED 3 times

for _ in range(3):

show_indicator(colour)

time.sleep_ms(400)

show_indicator(None)

time.sleep_ms(300)

# Keep LED on

show_indicator(colour)

time.sleep_ms(1500)    # pause before shuffle

winning = shuffle(colour)

# Tell ESP32 B which colour and slot won

send_msg("SKILL:" + colour + "," + str(winning + 1))

# Wait for result from ESP32 B

result = wait_result(8000)

show_indicator(None)

print("Spin again!\n")

else:

print("No card detected - try again!")

time.sleep_ms(50)