Project Design Document

Candy Shuffle Game — ESP32 Candy Dispenser

1. Project Overview

The Candy Shuffle Game is a Squid Game-themed interactive candy dispenser arcade built on two ESP32 microcontrollers. Players spin a colour wheel, track a shuffling LED sequence, and reach into the correct slot to win candy. A host-controlled Luck Mode triggers a jackpot dispensing all three slots at once via a smartphone app.

2. System Architecture

The system is split across two ESP32 boards communicating wirelessly:

ESP32 A:  Game controller — handles RFID, shuffle animation, indicator LEDs, start button

ESP32 B:  Dispenser unit — handles servos, IR sensors, NeoPixel ring, BLE for phone connection

Comms:  ESP32 A → ESP32 B: ESP-NOW (wireless, no wires between boards)

BLE:  Phone → ESP32 B: Bluetooth LE UART (MIT App Inventor)

3. Hardware Components

| Component | Quantity | Role | Board |
| --- | --- | --- | --- |
| ESP32-WROOM-32 DevKit | 2 | Main microcontrollers | A & B |
| RFID RC522 | 1 | Reads colour tag on spin wheel | A |
| Indicator LEDs (Yellow/Green/Blue) | 3 | Show detected colour | A |
| Slot LEDs | 3 | Shuffle animation per slot | A |
| Push Button | 1 | Player presses START | A |
| Servo Motors (SG90) | 3 | Open candy chutes on WIN | B |
| IR Sensors | 3 | Detect player hand in slot | B |
| NeoPixel WS2812B Ring (16 LED) | 1 | Visual feedback — colour/WIN/LOSE | B |
| Passive Buzzer | 1 | Audio feedback (currently disabled) | B |
| 330Ω Resistors | 3 | Current limit for indicator LEDs | A |

4. GPIO Pin Mapping

ESP32 A

| GPIO | Component | Mode |
| --- | --- | --- |
| GPIO 5 | Start Button | IN, PULL_UP |
| GPIO 18 | SPI SCK (RFID) | SPI2 |
| GPIO 19 | SPI MISO (RFID) | SPI2 |
| GPIO 23 | SPI MOSI (RFID) | SPI2 |
| GPIO 4 | RFID RST | OUT |
| GPIO 21 | RFID CS | OUT |
| GPIO 25 | LED Indicator Yellow | OUT |
| GPIO 26 | LED Indicator Green | OUT |
| GPIO 27 | LED Indicator Blue | OUT |
| GPIO 13 | Slot LED 1 | OUT |
| GPIO 14 | Slot LED 2 | OUT |
| GPIO 15 | Slot LED 3 | OUT |

ESP32 B

| GPIO | Component | Mode |
| --- | --- | --- |
| GPIO 25 | Servo 1 | PWM 50Hz |
| GPIO 26 | Servo 2 | PWM 50Hz |
| GPIO 33 | Servo 3 | PWM 50Hz |
| GPIO 4 | IR Sensor 1 | IN, PULL_UP (active LOW) |
| GPIO 13 | IR Sensor 2 | IN, PULL_UP (active LOW) |
| GPIO 14 | IR Sensor 3 | IN, PULL_UP (active LOW) |
| GPIO 5 | NeoPixel Data | OUT |
| GPIO 18 | Buzzer | OUT (active LOW) |

5. Communication Protocol

ESP-NOW messages between boards:

| Direction | Message | Trigger |
| --- | --- | --- |
| A → B | START | Player presses button |
| A → B | COLOR:<colour> | RFID detects wheel colour |
| A → B | SKILL:<colour>,<slot> | Shuffle complete — reveals winning slot |
| B → A | WIN | Player chose correct slot |
| B → A | LOSE | Player chose wrong slot |
| B → A | TIMEOUT | No hand detected in 5 seconds |
| Phone → B | WIN | Host triggers Luck Mode via BLE |

6. Game Modes

Skill Mode

Player spins colour wheel → presses START → RFID reads colour → NeoPixel syncs colour → slot LEDs shuffle (fast → slow → fast → blackout) → player reaches into a slot → WIN (servo opens, NeoPixel green) or LOSE (NeoPixel red).

Luck Mode

Host taps WIN in MIT App on phone → BLE signal to ESP32 B → all 3 servos fire simultaneously → NeoPixel gold flash → jackpot.