Test Plans & Results

Candy Shuffle Game — ESP32 Candy Dispenser

1. Component Tests — ESP32 B

| Test | Method | Expected | Result |
| --- | --- | --- | --- |
| NeoPixel lighting | np_fill(r,g,b) calls in test script | All 16 LEDs light correct colour | PASS |
| Servo open/close | Trigger dispense() per servo | Servo rotates to open then closes | PASS |
| IR sensor detection | Wave hand under each sensor | Active LOW triggers (value==0) | PASS (after GPIO12 fix) |
| Buzzer beep | Call beep() function | Short beep, silent otherwise | PASS (after HIGH fix) |
| ESP-NOW receive | Send test message from A | B prints received message | PASS |
| BLE advertising | Scan with nRF Connect app | Device appears as CandyGame | PASS |
| BLE connection | MIT App connect | App shows Connected status | In Progress |

2. Component Tests — ESP32 A

| Test | Method | Expected | Result |
| --- | --- | --- | --- |
| RFID tag read | Place each colour tag on reader | Correct colour string returned | PASS |
| Button debounce | Press START button | Single trigger, no bounce | PASS |
| Shuffle animation | Run shuffle() function | Fast→slow→fast→blackout sequence | PASS |
| Indicator LEDs | show_indicator(colour) calls | Correct LED lights per colour | PASS |
| ESP-NOW send | Trigger send_msg() | B receives message within 100ms | PASS |

3. Integration Tests

| Test | Steps | Expected | Result |
| --- | --- | --- | --- |
| Colour sync | Scan RFID tag on A → check B NeoPixel | NeoPixel lights correct colour immediately | PASS |
| Skill Mode — WIN | Scan tag → shuffle → reach correct slot | Green NeoPixel + servo dispenses | PASS |
| Skill Mode — LOSE | Reach wrong slot | Red NeoPixel, no dispensing | PASS |
| Skill Mode — TIMEOUT | Don't reach any slot for 5s | Orange flash, timeout message sent to A | PASS |
| Luck Mode | Send WIN from MIT App | All 3 servos fire + gold NeoPixel | Pending BLE fix |

4. Known Issues & Fixes Applied

| Issue | Root Cause | Fix Applied |
| --- | --- | --- |
| Flash corruption (invalid header: 0xffffffff) | GPIO12 is ESP32 strapping pin — PULL_UP held it HIGH at boot, switching flash voltage to 1.8V | Moved IR sensor 2 from GPIO12 to GPIO13 |
| Buzzer rings on startup | Pin(18, OUT) defaults LOW at boot; active LOW buzzer activates immediately | Added buzzer.value(1) immediately after pin definition |
| NeoPixel not syncing with colour | ESP32 B only received colour info after shuffle ended | Added COLOR:x ESP-NOW message sent immediately on RFID read |
| IR never triggered | Code checked value()==1 but PULL_UP holds HIGH; object = LOW | Changed detection to value()==0 |
| Flashing failed (Packet content transfer stopped) | Connected hardware interfered with SPI flash pins during esptool | Disconnect ALL hardware before flashing |
| Yellow/Blue NeoPixel swapped | Colour-to-RGB mapping in wrong order | Swapped YELLOW and BLUE RGB values in COLOUR_RGB dict |